ADJ3 Brewer computer software help

- Setup the software in your computer.

  - Copy all the ADJ3 brewer computer software files to your computer.

  - Open the ADJ3 computer software folder, then double click the setup.exe file or select the file and right click with your mouse, select Run as administrator.

  - When the software start to install, a welcome pop message display on your computer, click OK button to continue.

  - Click the Change Directory button to change the setup install folder or leave as default folder.

  - Click the setup icon start to continue install the software.

  - The software will list the program list ,then click the continue button.

  - When setup up completed ,Click the ok button to finish setup the ADJ3 brewer computer software. And it will add the program list.

Win64: X/Program Files (x86)/ADJ3

Win32: X/Program Files/ADJ3

X is the setup disk

Note: the best way is sent the exe file to the desktop.

- Install the USB driver for the computer.

  - Turn off the ADJ3 brewer switch, then connect the USB to the board and your computer.

  - When connect the computer, it will display the installing device driver software, until finish installing the driver.

  - Select computer on the desktop, then right click to select Manage.

  - Click the Device Manager , on the left part ,then click the Ports ( COM & LPT), you can see a USB-to –Serial Com Port , please remember the number ,it will use  later.

- Use computer to set your brewer

When finished installing assemble the driver, connect the PC and brewer the brewer’s LCD will display “PC Connected “

  - Interface

- computer connect area

- LCD display change area

- Buttons setting area

- Read message from brewer & Send setting to Brewer.

- Re-calibration flowmeter

- Saving and loading the program data

- Drip time change area

- Pump speed setting area (not design for now).

  - Use a USB cable to connect the computer and the brewer. ( Please make sure the brewer power off).

  - Select the correct COM ports (check on the device manager ports list), then click connect. When connect is successful, it will has a pop message.

  - Change LCD display of the brewer.

    - Text the words on the LCD display change area (top line of the blue part). Do not text the words over 16 letters.

    - Finish the change the LCD display change, click the Send to Brewer button to send the data to brewer.

      - If the brewer beeps two sounds , the light near the button color is green and has a pop message to tell you send successful.

      - If the light is red it means send message fail, click the button again until the light change to green and has a pop message display on your computer.

Note: you can send the data with the button setting together. Or loading from the program.

  - Change the button setting.

    - Click the Read from Brew button to get the current setting from brewer, when get the data successful, the light is green, and a pop message display on your computer. (optional step )

    - Text a number in the volume total blank area (pink color part) to setting the total volume of the button. The Max setting is 102 Oz, and any input number is over 102 Oz it will change to the Max setting 102Oz.

    - Click the default button of the volume area ( near the total volume part) to reset the button default volume setting, the Left button is 64 Oz, Center button is 74 Oz, and right button is 84 Oz.

    - Input a number in the each button pulse brewing setting part, the machine will do the pulse brewing at the setting. Can’t setting a number over the button total volume.

    - Click the send to brewer button to save the setting, and make sure the light is green and has the pops message to indicator

Note: To verify the setting is correct or not, need re power the board, then open the software and click the read from brewer to verify. Directly to click the ready from brewer button the volume and pulse brew setting is not correct.

  - Disable and enable the buttons and set LCD display total brewing cup.

    - Simple click the disable button to disable the button. And on the Write part it will display OFF.

Note: the Left button can’t be disabled.

    - Put any number in the Brew Count write box of the button, it will enable the button and display the number on the button total brewing cup in the LCD.

  - recalibration flowmeter.

    - Use the PC software to connect the brewer and read data from the brewer. And record the flowmeter default setting volume per Oz.

    - Suggest set the #1 (left) button total volume is 10 Oz and without any pulse. The volume setting bigger can get more accurate data.

    - Dis-connect the brewer, and turn on the brewer to the normal operation. And when the brewer display “ Ready to Brew” .

    - Place a container to a scale and zero the weight. And set the scale’s unit to Oz.

    - Place the container under the brew chamber and press the #1 button to dispense the hot water to container. Use a scale to measure the water volume and write the data. Don’t measure the water come from the vent pipe, it will affect the setting.

    - Suggest to repeat the 3.7.4 and 3.7.5 at least 5 times, count the average volume and write down the data. More times is can get more accurate data.

    - Recalibration the flowmeter sign number use the below formula.

Calibration 1 Oz= (set volume* flowmeter default signs number per oz) /average water volume (Oz)

Such as set the volume is10 Oz, and default flowmeter signs per oz is 88, and get the average volume is 9 Oz, so the new signs per Oz=(10*88)/ 9 =97.8 signs.

    - turn off the brewer and use the USB to connect the brewer and computer. Open the software to access the brewer.

    - Put the data to the new setting part and set all the other setting you want. Send the data to the brewer successful. The brewer will use the new setting .

Important : the flowmeter setting is only can use one decimals only, any wrong put machine will reset to factory initial setting. such as 90.5 is acceptable, but 90.51 is incorrect.

The flowmeter setting volume range is 70-105, any number is over the range it will change to factory default 88.

If need more accuracte ,set the volume bigger and brew more times .

3.7.10 the flowmeter sign number will not save to the program data. When calculate the flowmeter sign, the machine different the sign number maybe different.

  - Change the drip time. In the computer software we can easy to change the drip out time. The range is from 70-105 seconds. Default time is 90 seconds.

  - Change the pump speed setting. in the computer software we can change the pump setting, the range is from 70% to 100%. Default is 100%. And it is designed for the future use only.

  - Save the program setting data.

When you finish your setting, you can simple to click the save button to save the setting to the file ADJ3 PROGRAM DATA.TXT . The file is in the computer software instead folder or user/username/AppData/Local/VirtualStore/Program Files/ADJ3 in the 64bit system it is should be on user/username/AppData/Local/VirtualStore/Program Files (X86)/ADJ3. Please don’t change any setting in the file. You can save different ratio, but when save one setting, you need to copy the ADJ3 PROGRAM DATA.TXT to other folder and rename it.

  - load the program data

simple to click the load button, it will load the save setting in the software ,then click send to Brewer to machine.